# Product specification: Acme R7 warehouse robot

# Overview

The R7 is an autonomous mobile robot for warehouse picking. It navigates with LiDAR and stereo cameras and integrates with common warehouse management systems through a REST API.

# Battery and charging

The R7 uses a 48 V lithium iron phosphate battery with a capacity of 2.4 kWh. A full charge takes 90 minutes on the docking station; opportunity charging to 80% takes 45 minutes. Typical runtime is 10 hours in mixed operation.

# Safety

The robot is certified to ISO 3691-4. It stops within 0.3 m at full speed when an obstacle is detected. Emergency stop buttons are located on all four sides. The maximum speed in shared zones with people is limited to 1.0 m/s.

# Maintenance

Wheels and brushes should be inspected every 500 operating hours. The LiDAR window must be cleaned weekly. Firmware updates are delivered over the air every quarter.

# Warranty

The R7 comes with a 24-month warranty covering parts and labour. The battery is covered for 3,000 charge cycles or 36 months, whichever comes first. Damage from misuse is excluded.

# Technical data

| Parameter | Value |
| --- | --- |
| Payload | 600 kg |
| Maximum speed | 2.0 m/s |
| Dimensions (L x W x H) | 1100 x 750 x 320 mm |
| Weight | 185 kg |
| Navigation | LiDAR + stereo cameras, no floor markers needed |
| IP rating | IP54 |
| Operating temperature | 0 to 40 °C |